MAXIMUM NUMBERS PERMITTED IN HALL

CROSBY RAVENSWORTH - HALL CAPACITY (Revised 17/06/24)

| Table 1 | Table 2 |
| --- | --- |

| Maximum No. of Persons | Minimum Number of escape routes/exits |  | A width of at least 750 mm and 1050mm can accommodate up to: |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Max No. of Persons |  | Type of Premises |
|  |  |  | 750mm | 1050mm |  |
| 60 | 1 |  | 80 | 160 | In higher risk premises |
| 600 | 2 (there MUST be more than one escape route from halls) |  | 100 | 200 | In normal risk premises |
| More than 600 | 3 |  | 120 | 240 | In lower risk premises |
|  |  |  | An additional 75mm should be allowed for each additional 15 persons (or part of 15). |  |  |

Note:	Doors that do NOT open in the direction of travel i.e. open inwards, are only suitable for a maximum 60 people.

Doors that are classified as suitable for disabled persons should be a minimum of 1200mm in width

Table 3

| Hall Dimensions |  | 92.4 m² |
| --- | --- | --- |
| BS 9999: 2017 | General Assembly/Standing - Occupant Capacity based on floor space factor of 0.5m²/ person | 184 Persons (now take into account No., size & design of escape routes/exits – see below) |
|  | Seated at tables - Occupant Capacity based on floor space factor of 1.0m²/person | 92 Persons (now take into account No., size & design of escape routes/exits – see below) |
|  | Seated in rows in loose seats in rows - Occupant Capacity based on floor space factor of 0.75m²/ person | 123 Persons (now take into account No., size & design of escape routes/exits – see below) |
| Number of Escape Routes/Exits |  | 4 |
| Width of Escape Routes/Exits and Direction of Travel |  | - Gap/corridor to left leading to final exit with push bar handle (1050mm) which opens in direction of travel (DOT). - Gap to right leading to final exit door in KS2 art area (790mm) (DOT). - Door (810mm) leading to foyer door (900mm) and final exit at main entrance (1600mm) – all open DOT. - Inward opening door (830mm) leading to final exit in store room/passageway (1800mm) (DOT). |

Calculations (from Table 3)

Determine no. and width of escape routes (see Table 2) and discount the largest exit as though it were blocked by fire i.e.

- Gap/corridor to left leading to final exit with push bar handle (1050mm) which opens in direction of travel (DOT).  As the largest exit, this has been discounted for calculation purposes as though blocked due to fire.

- Gap to right leading to final exit door in KS2 art area (790mm) (DOT).  This route is suitable for 100 people to use.

- Door (810mm) leading to foyer door (900mm) and final exit at main entrance (1600mm) – all open DOT.  This route is suitable for 100 people to use.

- Inward opening door (830mm) leading to final exit in store/passageway (1800mm) (DOT).  As the inner door opens inwards, this route is only suitable for 60 people to use.  Ensure the route through the store/passageway is always kept completely clear.

| HALL CAPACITY: In line with BS 9999: 2017, our Hall Capacity is: |  |  |
| --- | --- | --- |
| Under Normal, Day to Day Circumstances (figures include everyone present): |  |  |
| Maximum No. of people permitted in hall (Standing) OR | 160 |  |
| Maximum No. of people permitted in hall (Seated at Tables) OR | 92 |  |
| Maximum No. of people permitted in hall (Seated in Loose Seats in Rows) | 123 |  |
| During Performances in front of an Audience with 30 Children involved (figures include everyone present): |  |  |
| Maximum No. of children (C) & staff (S) involved in performance in hall (Standing) PLUS | (C) 30 | (S) 5 |
| Maximum No. of parents (audience) permitted in hall (Seated in Loose Seats in Rows) OR | 99 |  |
| Maximum No. of parents (audience) permitted in hall (Seated at Tables – cabaret style) | 74 |  |